ATIVIDADE 2.1

Diagrama de blocos para os itens a,b,c e d:

Tabela verdade para os itens a,b,c e d:

CÓDIGOS:

Código item 2.1 a:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity demux_1x4 is

Port (

entrada : in  STD_LOGIC_VECTOR(2 downto 0);

seletor : in  STD_LOGIC_VECTOR(1 downto 0);

saida0  : out STD_LOGIC_VECTOR(2 downto 0);

saida1  : out STD_LOGIC_VECTOR(2 downto 0);

saida2  : out STD_LOGIC_VECTOR(2 downto 0);

saida3  : out STD_LOGIC_VECTOR(2 downto 0)

);

end demux_1x4;

architecture mapeamento of demux_1x4 is

begin

saida0 <= entrada when seletor = "00" else "000";

saida1 <= entrada when seletor = "01" else "000";

saida2 <= entrada when seletor = "10" else "000";

saida3 <= entrada when seletor = "11" else "000";

end mapeamento;

Código item 2.1 b:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity demux_1x4 is

Port (

entrada : in  STD_LOGIC_VECTOR(2 downto 0);

seletor : in  STD_LOGIC_VECTOR(1 downto 0);

saida0  : out STD_LOGIC_VECTOR(2 downto 0);

saida1  : out STD_LOGIC_VECTOR(2 downto 0);

saida2  : out STD_LOGIC_VECTOR(2 downto 0);

saida3  : out STD_LOGIC_VECTOR(2 downto 0)

);

end demux_1x4;

architecture mapeamento of demux_1x4 is

begin

with seletor select

saida0 <= entrada when "00",

"000"   when others;

with seletor select

saida1 <= entrada when "01",

"000" when others;

with seletor select

saida2 <= entrada when "10",

"000" when others;

with seletor select

saida3 <= entrada when "11",

"000" when others;

end mapeamento;

Código item 2.1 c:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity demux_1x4 is

Port (

entrada : in  STD_LOGIC_VECTOR(2 downto 0);

seletor : in  STD_LOGIC_VECTOR(1 downto 0);

saida0  : out STD_LOGIC_VECTOR(2 downto 0);

saida1  : out STD_LOGIC_VECTOR(2 downto 0);

saida2  : out STD_LOGIC_VECTOR(2 downto 0);

saida3  : out STD_LOGIC_VECTOR(2 downto 0)

);

end demux_1x4;

architecture mapeamento of demux_1x4 is

begin

process(entrada, seletor)

begin

if seletor = "00" then saida0 <= entrada;

elsif seletor = "01" then saida1 <= entrada;

elsif seletor = "10" then saida2 <= entrada;

elsif seletor = "11" then saida3 <= entrada;

end if;

end process;

end mapeamento;

Código item 2.1 d:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity demux_1x4 is

Port (

entrada : in  STD_LOGIC_VECTOR(2 downto 0);

seletor : in  STD_LOGIC_VECTOR(1 downto 0);

saida0  : out STD_LOGIC_VECTOR(2 downto 0);

saida1  : out STD_LOGIC_VECTOR(2 downto 0);

saida2  : out STD_LOGIC_VECTOR(2 downto 0);

saida3  : out STD_LOGIC_VECTOR(2 downto 0)

);

end demux_1x4;

architecture mapeamento of demux_1x4 is

begin

process(entrada, seletor)

begin

case seletor is

when "00" => saida0 <= entrada;

when "01" => saida1 <= entrada;

when "10" => saida2 <= entrada;

when "11" => saida3 <= entrada;

when others => null;

end case;

end process;

end mapeamento;

Plano de simulação usado nos items 2.1 a,b,c e d:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_demux_1x4 is

end tb_demux_1x4;

architecture simulacao of tb_demux_1x4 is

component demux_1x4

Port (

entrada : in  STD_LOGIC_VECTOR(2 downto 0);

seletor : in  STD_LOGIC_VECTOR(1 downto 0);

saida0  : out STD_LOGIC_VECTOR(2 downto 0);

saida1  : out STD_LOGIC_VECTOR(2 downto 0);

saida2  : out STD_LOGIC_VECTOR(2 downto 0);

saida3  : out STD_LOGIC_VECTOR(2 downto 0)

);

end component;

signal t_entrada : STD_LOGIC_VECTOR(2 downto 0);

signal t_seletor : STD_LOGIC_VECTOR(1 downto 0);

signal t_saida0  : STD_LOGIC_VECTOR(2 downto 0);

signal t_saida1  : STD_LOGIC_VECTOR(2 downto 0);

signal t_saida2  : STD_LOGIC_VECTOR(2 downto 0);

signal t_saida3  : STD_LOGIC_VECTOR(2 downto 0);

begin

UUT: demux_1x4 port map (

entrada => t_entrada,

seletor => t_seletor,

saida0  => t_saida0,

saida1  => t_saida1,

saida2  => t_saida2,

saida3  => t_saida3

);

simular: process

begin

-- Usando 101 na  entrada para ver para onde ele vai quando mudamos o seletor

t_entrada <= "101";

-- Cenario 1: seletor em 00 (a saida0 deve ligar e mostrar 101, as outras ficam em 000)

t_seletor <= "00";

wait for 10 ns;

-- Cenario 2: seletor em 01 (A saida1 deve ligar e mostrar 101)

t_seletor <= "01";

wait for 10 ns;

-- Cenario 3: seletor em 10 (a saida2 deve ligar e mostrar 101)

t_seletor <= "10";

wait for 10 ns;

-- Cenario 4: seletor em 11 (A saida3 deve ligar e mostrar 101)

t_seletor <= "11";

wait for 10 ns;

wait for 10 ns;

wait;

end process;

end simulacao;